## Setup Notes

This package includes a GetIrSetup.msi that requires Microsoft Windows Installer to install it (available as a download from Microsoft). The GetIRSetup will install a COM dll called GetIR4.dll which you can uninstall using the Add Remove Programs application within Windows Control Panel. The DLL and sample Excel spreadsheet has been testing running in Windows 2000 and XP.

To use, create the simple circuit detailed below or refer to the originator of the circuit Igor Cesko as per the attached “Remote Control of a Computer.mht to find alternative circuits and suppliers of them. As can be seen, the circuit simply requires a resister, capacitor, Zener diode and 3 pin IR optical demodulator. These can be simply directly wired to the 9 pin RS232 D connector and should cost less than 5 USD.

### Note:

All common IR detectors can be used. To create the circuit, refer to your ID detector’s specification sheet to find out the pins for the OUT, VCC and GND connections, and modify the circuit below accordingly.

After attaching the device to your Com port, simply run the sample Excel spreadsheet, edit the input table for your particulars, click on the GetIr button, and press a button on your remote. If everything is working, you should get a list of times in the “A” column.

## Background

I was trying to train my universal remote to understand my airconditioner IR signals, but as the signal was too long, I needed an alternative method to learn it. I came across Igor Cesko’s web page in which he has designed a device and software to remotely control a PC (with the help of the Girder software). Now, his device (see sample above) simply decodes an IR signal and manipulates the serial port’s DSR line. I then wrote an interrupt driven .NET Visual C++ COM object to monitor and time the serial port’s DSR lines. The reason for using a COM dll was to allow it to be called from any language including VBA (MSOffice, Excel…), VBScript and JavaScript.

## Calling Details

The call is (VBA format):

Dim obj As New CGetIR

Call obj.GetIr(Port, LearnTime, IOTimeout, PosAdj, NegAdj, Result, NumRes, Arr)

As defined by:

Sub GetIr(Port As String, LearnPeriod As Long, EvtTimeout As Long, PosAdj As Long, NegAdj As Long, ByRef Result As Long, ByRef NumRes As Long, ByRef  Arr() as long)

This is a member of class: GetIr4.CGetIR

Where:

Port: is the serial port eg. COM1:

LearnPeriod: is the number of seconds you want the system to wait for your remote’s IR signal to be sent eg. 5

EvtTimeout: is the number of milliseconds that the system will wait before the serial port’s interrupt will time out.  Eg. 2500

PosAdj: This is to overcome the interrupt triggering too early. Use zero against known timebase and then adjust this value to correct the +ve time base. Eg. -120

NegAdj: This is to overcome the interrupt triggering too early. Use zero against known timebase and then adjust this value to correct the -ve time base. Eg. 170

Result: is the result of the call. If it is zero, then all is well. See below for other errors.

NumRes: These are the resulting number of time signals received.

Arr: This array will contain the IR signal times

Possible result errors are:

0: No error

-1: Cannot open COM port (normally caused by incorrect port name)

-2: Cannot set com interrupt mask

-3: Cannot create event

-4: WaitForSingleObject failure

-5: Cannot get Overlapped result

-10: Cannot build DCB for serial device

-11: No high performance counter available within Windows for your hardware

-12: Cannot set com state

6: Cannot lock Array passed to GetIR for writing (invalid arg)

7: Cannot lock Array passed to GetIR for writing (unexpected lock)

13: Incorrect array passed to GetIR

Where the italic items are the ones normally caused by incorrect parameters to the subroutine.

All queries should be posted in the jp1 group

Regards,

Haggis